# Automotive Body Fit Process Capability Study

High-Volume Automotive OEM | Hood-to-Fender Gap | Synthetic Educational Case Study

PORTFOLIO DISCLAIMER — This project uses a synthetic educational dataset created solely to demonstrate statistical process capability analysis. It does not contain proprietary or confidential manufacturing data from any company.

| Samples | 100 | Mean | 3.500000 mm |
| --- | --- | --- | --- |
| Sample STDEV | 0.196000 mm | Specification | 3.000 to 4.000 mm |
| Cp | 0.850 | Cpk | 0.850 |
| Observed rejects | 1 (10,000 ppm) | Expected rejects* | 10,741 ppm |

# 1. Executive Summary

The study evaluates 100 synthetic hood-to-fender gap measurements. The mean is 3.500 mm against a 3.500 mm target, so the process is centered. The sample standard deviation is 0.196 mm. Cp and Cpk are both 0.850, below the illustrative 1.33 capability criterion. The process is aimed correctly, but variation is too large.

Conclusion: Not capable. The estimated natural process spread is wider than the available specification window.

# 2. Definitions and Equations

Cp = (USL - LSL) / (6 x s)

Cpk = min[(USL - mean) / (3 x s), (mean - LSL) / (3 x s)]

For this dataset, Cp = (4.000 - 3.000) / (6 x 0.196) = 0.850. Because the mean is centered, Cpu = Cpl = Cpk = 0.850. Calculations use the sample standard deviation (n - 1).

# 3. Visual Analysis

Figure 1. Capability histogram and fitted normal distribution.

Figure 2. Time-ordered measurements; sample 31 exceeds the USL.

# 4. Engineering Interpretation

Centering is not the primary problem: the mean matches the nominal target.

Variation is the primary problem: six standard deviations equal 1.176 mm, wider than the 1.000 mm specification window.

One observed measurement is out of specification, equivalent to 10,000 ppm in this small dataset.

The normal-model expected PPM is an estimate, not a guarantee of future defect rate.

Confirm statistical stability with an appropriate control chart and measurement adequacy with Gauge R&R before relying on capability indices.

# 5. Recommended Improvement Plan

| Workstream | Recommended action |
| --- | --- |
| Measurement | Perform Gauge R&R and standardize gauge orientation and contact pressure. |
| Fixture | Inspect locators, clamps, hinge interfaces, and wear points; establish preventive-maintenance limits. |
| Method | Standardize adjustment sequence, torque method, and acceptance criteria in the work instruction. |
| Material/design | Review tolerance-stack contributors and supplier variation for closures, hinges, fenders, and mounts. |
| Control | Use rational subgroups and an X-bar/R chart, or an I-MR chart when subgrouping is not appropriate. |
| Verification | After corrective action, collect an independent time-ordered dataset and demonstrate the program-specific Cpk requirement. |

# 6. Statistical Cautions

*Expected PPM assumes an independent, stable, approximately normal process. Cp/Cpk do not establish stability, normality, causal control, or measurement-system adequacy. Specification limits are engineering requirements, not control limits. Acceptance thresholds must follow applicable customer and program requirements.

# 7. References

AIAG Statistical Process Control (SPC), 2nd Edition

AIAG Measurement Systems Analysis (MSA), 4th Edition

AIAG & VDA FMEA Handbook

Douglas C. Montgomery, Introduction to Statistical Quality Control, 8th Edition

ISO 22514 series — Statistical methods in process management

# Appendix A — Raw Measurements

| Sample | Value (mm) | Status |
| --- | --- | --- |
| 1 | 3.589583 | PASS |
| 2 | 3.250239 | PASS |
| 3 | 3.702068 | PASS |
| 4 | 3.750044 | PASS |
| 5 | 3.020328 | PASS |
| 6 | 3.184072 | PASS |
| 7 | 3.544947 | PASS |
| 8 | 3.432880 | PASS |
| 9 | 3.508446 | PASS |
| 10 | 3.297414 | PASS |
| 11 | 3.734608 | PASS |
| 12 | 3.708967 | PASS |
| 13 | 3.529349 | PASS |
| 14 | 3.797153 | PASS |
| 15 | 3.630665 | PASS |
| 16 | 3.295837 | PASS |
| 17 | 3.605743 | PASS |
| 18 | 3.270705 | PASS |
| 19 | 3.734369 | PASS |
| 20 | 3.500087 | PASS |
| 21 | 3.466035 | PASS |
| 22 | 3.340848 | PASS |
| 23 | 3.821203 | PASS |
| 24 | 3.473689 | PASS |
| 25 | 3.404594 | PASS |
| 26 | 3.423822 | PASS |
| 27 | 3.647018 | PASS |
| 28 | 3.604908 | PASS |
| 29 | 3.616842 | PASS |
| 30 | 3.621407 | PASS |
| 31 | 4.053146 | FAIL |
| 32 | 3.410124 | PASS |
| 33 | 3.383418 | PASS |
| 34 | 3.307325 | PASS |
| 35 | 3.668133 | PASS |
| 36 | 3.797590 | PASS |
| 37 | 3.483930 | PASS |
| 38 | 3.300666 | PASS |
| 39 | 3.304622 | PASS |
| 40 | 3.676868 | PASS |
| 41 | 3.700251 | PASS |
| 42 | 3.649755 | PASS |
| 43 | 3.344740 | PASS |
| 44 | 3.571273 | PASS |
| 45 | 3.542132 | PASS |
| 46 | 3.567874 | PASS |
| 47 | 3.732597 | PASS |
| 48 | 3.569112 | PASS |
| 49 | 3.684015 | PASS |
| 50 | 3.529740 | PASS |
| 51 | 3.585647 | PASS |
| 52 | 3.671996 | PASS |
| 53 | 3.144962 | PASS |
| 54 | 3.432015 | PASS |
| 55 | 3.393984 | PASS |
| 56 | 3.351461 | PASS |
| 57 | 3.443252 | PASS |
| 58 | 3.889945 | PASS |
| 59 | 3.294187 | PASS |
| 60 | 3.757038 | PASS |
| 61 | 3.088002 | PASS |
| 62 | 3.428175 | PASS |
| 63 | 3.553758 | PASS |
| 64 | 3.660623 | PASS |
| 65 | 3.692169 | PASS |
| 66 | 3.712893 | PASS |
| 67 | 3.424683 | PASS |
| 68 | 3.396008 | PASS |
| 69 | 3.729202 | PASS |
| 70 | 3.464409 | PASS |
| 71 | 3.190757 | PASS |
| 72 | 3.226693 | PASS |
| 73 | 3.280656 | PASS |
| 74 | 3.638148 | PASS |
| 75 | 3.548628 | PASS |
| 76 | 3.686935 | PASS |
| 77 | 3.404866 | PASS |
| 78 | 3.552695 | PASS |
| 79 | 3.670558 | PASS |
| 80 | 3.434620 | PASS |
| 81 | 3.627956 | PASS |
| 82 | 3.345644 | PASS |
| 83 | 3.421067 | PASS |
| 84 | 3.416352 | PASS |
| 85 | 3.210907 | PASS |
| 86 | 3.635577 | PASS |
| 87 | 3.394229 | PASS |
| 88 | 3.515839 | PASS |
| 89 | 3.634006 | PASS |
| 90 | 3.625371 | PASS |
| 91 | 3.680601 | PASS |
| 92 | 3.487832 | PASS |
| 93 | 3.405864 | PASS |
| 94 | 3.492568 | PASS |
| 95 | 3.086875 | PASS |
| 96 | 3.147497 | PASS |
| 97 | 3.178893 | PASS |
| 98 | 3.261024 | PASS |
| 99 | 3.613572 | PASS |
| 100 | 3.284182 | PASS |