# Manufacturing Process Excellence Toolkit

Integrated Python Portfolio: Capability | PFMEA | OEE | Gauge R&R | SPC | Process Improvement

PUBLIC PORTFOLIO DISCLAIMER — This portfolio uses synthetic educational datasets and generalized manufacturing case studies solely to demonstrate engineering, statistical analysis, and problem-solving methods. It contains no proprietary production records, confidential specifications, internal documents, or company-specific manufacturing data.

# 1. Executive Summary

This report demonstrates an integrated manufacturing-engineering workflow. Capability evaluates output variation; PFMEA prioritizes preventive action; OEE quantifies production losses; Gauge R&R evaluates measurement adequacy; process-data tools expose instability and dominant losses; generalized case studies show how these methods support hands-on line ownership and technical problem solving.

# 2. Process Capability

| Metric | Result |
| --- | --- |
| Samples | 100 |
| Mean | 3.500000 mm |
| Sample standard deviation | 0.196000 mm |
| Cp / Cpk | 0.850 / 0.850 |
| Observed nonconformance | 1 (10,000 ppm) |
| Interpretation | Not capable |

# 3. PFMEA Risk Review

RPN is calculated as Severity x Occurrence x Detection. Portfolio priority is a transparent educational triage and is not the proprietary AIAG-VDA Action Priority table.

| Process step | Failure mode | RPN | Revised RPN |
| --- | --- | --- | --- |
| Fixture locate | Panel located incorrectly | 200 | 48 |
| Gap measurement | Incorrect measurement | 180 | 36 |
| Final verification | Defect not detected | 168 | 42 |
| Fastener torque | Joint under-torqued | 72 | 32 |
| Material presentation | Wrong component variant | 42 | 14 |

# 4. OEE Trend

| Date | Availability | Performance | Quality | OEE |
| --- | --- | --- | --- | --- |
| 2026-07-01 | 87.9% | 91.0% | 95.3% | 76.2% |
| 2026-07-02 | 90.6% | 92.4% | 96.7% | 81.0% |
| 2026-07-03 | 91.9% | 93.6% | 98.0% | 84.2% |
| 2026-07-04 | 93.5% | 94.3% | 98.6% | 87.0% |
| 2026-07-05 | 94.6% | 95.2% | 98.9% | 89.0% |

# 5. Gauge R&R

| Metric | Result |
| --- | --- |
| Study design | 10 parts x 3 operators x 3 trials |
| Total Gauge R&R (% study variation) | 4.67% |
| Repeatability | 3.54% |
| Reproducibility | 3.04% |
| Number of distinct categories | 30 |
| Interpretation | Generally acceptable |

# 6. Process Data and Pareto

| Defect category | Count | Percent | Cumulative |
| --- | --- | --- | --- |
| Gap/flush | 42 | 42.0% | 42.0% |
| Surface appearance | 27 | 27.0% | 69.0% |
| Fastener | 16 | 16.0% | 85.0% |
| Electrical check | 9 | 9.0% | 94.0% |
| Material mismatch | 6 | 6.0% | 100.0% |

# 7. Hands-On Engineering and Problem Solving

Process launch: Converted an uncontrolled repair need into defined work content, tooling, measurement, standardized work, training, and production controls.

Line redesign: Used line observation, cycle-time analysis, task precedence, work balancing, layout, controls, and ramp support to improve flow and expose bottlenecks.

Vision commissioning: Supported installation, recipe development, correlation, false-call reduction, reaction plans, and ongoing verification with an equipment supplier.

Variation reduction: Connected MSA, time-ordered SPC, fixture inspection, standardized method, tolerance review, corrective action, and independent capability validation.

# 8. Professional Use and Limitations

Results are educational demonstrations. Production decisions require approved specifications, rational sampling, stable-process evidence, validated measurement systems, customer-specific acceptance criteria, and review by qualified process and quality personnel.